| # | Parameter | Requirement |
| --- | --- | --- |
| 1 | Roller Diameter | SG#1: ___ - 5 mm/+0 mm SG#2: ___ - 5 mm/+0 mm SG#3: ___ - 5 mm/+0 mm |
| 2 | Roller Face length | 2200 mm – 2500 mm (based on the line requirement) |
| 3 | Roller Run out | < 0.1 mm |
| 4 | DU shaft/journal roughness | < 0.4 µm & Shaft/Journal Surface is free from pitting, scratches or any damage. |
| 5 | DU Shaft diameter | 249.93 to 250.00 mm in 400mm & 340mm Roller |
|  |  | 209.93 to 210.00 mm in 260mm & 270 mm Roller. |
| 6 | The DU shaft mounting position | Should be perfectly round fit. |
| 7 | Circlip grooves | Should be damage free |
| 8 | Tapped holes threads | Should be damage free |